Team 1’s list of materials & components

| Component Name | Description | Price (approx.) | Motivation | Link |
| --- | --- | --- | --- | --- |
| Power Supply 12VDC 2A | AC/DC adapter providing 24W of power. | 9.50 € | Provides a stable, high-capacity power foundation with enough headroom to prevent overheating. | https://mauser.pt/035-2829/fonte-de-alimentacao-12vdc-2-0a-24w-5-5x2-1mm |
| Solenoid Valve 12VDC | Electrically operated water valve | 12.30 € | Allows for automated irrigation control directly managed by the microcontroller logic. | https://mauser.pt/096-8157/electrovalvula-solenoide-12vdc-450ma-g1-2-nc |
| Buck Converter | Step-down voltage regulator (12V to 5V). | 2.50 € | Efficiently powers the 5V logic components from the 12V source without the heat of linear regulators. | https://mauser.pt/096-8732/modulo-mini-conversor-step-down-uin-4-5-28v-uout-0-8-20v-3a-lm2596s-mp2307dn |
| ESP32 Dev Board | Dual-core microcontroller with integrated WiFi/Bluetooth working as "brain" of the system. | 9.90 € | Acts as the "brain" for smart processing and allows for remote data monitoring via the internet. | https://mauser.pt/096-8744/modulo-esp32-wroom-nodemcu-wifi-cp2102 |
| Relay Module (1-ch) | Electromagnetic switch for high-power loads. | 3.20 € | Provides electrical isolation, allowing the low-power ESP32 to safely trigger the 12V solenoid. | https://mauser.pt/096-4673/whadda-wpm406-modulo-de-1-rele-5v-compativel-com-arduino |
| Capacitive Soil Moisture Sensor | Analog sensor measuring soil water content. | 4.90 € | Uses capacitive pads rather than exposed metal to prevent corrosion, ensuring a much longer product life. | https://mauser.pt/096-7465/seeed-modulo-grove-sensor-de-humidade-capacitivo-resistente-a-corrosao |
| Temperature Sensor (DS18B20) | Waterproof stainless steel probe. | 5.80 € | The rugged, anti-corrosive casing is essential for reliable feedback in humid agricultural environments. | https://mauser.pt/096-8756/ds18b20-sensor-de-temperatura-100cm |
| Diode (1N4007) | General purpose rectifier diode. | 0.15 € | Functions as a flyback diode to protect sensitive electronics from inductive voltage spikes from the valve. | https://mauser.pt/007-0209/diodo-rectificador-1000v-1a-1n4007 |
| DC Jack Terminal | 2-pin screw terminal for DC power input. | 1.40 € | Ensures a secure, professional power connection that is easy to assemble or replace without soldering. | https://mauser.pt/011-2116/ficha-dc-femea-5-5x2-1mm-bloco-terminal-2pin-c-parafusos-2-pecas |
| Resistor 4.7k | Fixed value pull-up resistor. | 0.05 € | Enables to expand the circuit into plus and minus wires. | https://mauser.pt/104-7044/resistencia-de-filme-metalico-4-7kr-0-6w-1-2-5x6-8mm |
| SPST Slide Switch | Master switch | 1.38 € | Acts as a master kill switch for the entire system, allowing for a safe total shutdown without unplugging the power supply. | https://mauser.pt/010-1550/sci-botao-basculante-1-posicao-estavel-unipolar-off-on-spst-250v-10a |
| Red LED (5mm) | Standard indicator LED. | 0.36 € | Provides visual confirmation that the power is reaching the 5V logic side of your board. | https://mauser.pt/017-0973/optosupply-led-5mm-transparente-vermelho-1120-1560mcd-30-12v |
| Resistor 470 Ω | Resistor working with Red LED | 0.15 € | Drops the 5V from your Buck Converter to a safe level for the LED. | https://mauser.pt/104-4113/resistencia-fusivel-470r-1w |
| Sum |  | 51.70 € |  |  |